| Control Strategy | System/Platform | RL Algorithm | Reward Mechanism Features | Simulation/Hardware Validation | Performance Metrics | Source |
| --- | --- | --- | --- | --- | --- | --- |
| MP-NMPC (Multimodal Nonlinear Model Predictive Control) | Autonomous quadrotor | Multimodal PPO (concatenated depth/inertial features) | Rewards goal achievement and alignment between RL actions and NMPC; penalizes collisions. | Unreal Engine/Gazebo simulation and real-world flight in cluttered environments. | 100% success rate in waypoint reaching; 80% path optimality; superior generalization over standard DRL. | [1] |
| End-to-End Local Motion Planning | MAV quadrotor | Actor-Critic | Hybrid reward: shaped (flight time) and sparse (distance to goal); penalties for obstacle proximity (<1.0 m). | Gazebo simulation and real-world outdoor flight (NVIDIA Jetson TX2, Hokuyo lidar). | Safe navigation in unknown forests; robust against localization noise; smooth velocity and heading. | [2] |
| Synergetic Formation Control | Multi-UAVs (DJI Phantom 4) | Pairwise DRL (P-A3C, P-AC, P-DDQN) | Segmented rewards for UAV/obstacle safety and formation progress; uses short/mid/long-term detection. | Gazebo/Python simulation and hardware (STM32F303 MCU on DJI Phantom 4). | 96.3% success rate in dense formations; real-time performance of 17.14 Hz. | [3] |
| AirPilot (DRL-tuned nonlinear PID) | COEX Clover 4 autonomous drone | PPO | Encourages navigation speed, precision, quick settling time, and energy efficiency. | Gazebo simulator and real-world deployment with Vicon tracking system. | 90% reduction in navigation error; 21% improvement in speed; 17% reduction in settling time. | [4] |
| TD3-based angular momentum control | Rigid spacecraft with SGCMG array | TD3 | Hybrid reward penalizing attitude/angular velocity errors, state violations, and energy consumption. | OpenAI Gym simulator. | Large-angle maneuver (>60∘) convergence in <1 min; steady-state accuracy in MRP of 10−3. | [5] |
| TD3-HD (Dimension-Wise Clipping) | Rigid spacecraft with 4 reaction wheels | TD3 with HER | Penalizes attitude error and stability violations (∣w∣>1); rewards precision maintenance. | Basilisk Astrodynamics Simulation Framework. | Stable pointing after actuator failure at 3000s; peak attitude error within 0.25 limit. | [6] |
| Constraint-Aware RL (Smooth Maneuver Control) | Rigid-body spacecraft (1–3 DOF) | Tabular Q-learning or Policy Gradient | Structured reward (Rperf​+Rsmooth​) penalizing actuator switching and total control effort. | High-fidelity simulation with 3-DOF model, realistic latency, and stochastic disturbances. | Significant reduction in Actuator Total Variation (ATV); settling time within 2% of target. | [7-9] |
| Intelligent Impact Time Control Guidance (ITCG) | Missile intercepting maneuvering targets | PPO | Exponential reward minimizing guidance errors and energy consumption (acceleration integral). | Numerical 3D engagement simulations and 500 Monte Carlo runs. | Impact time error <0.13 s; miss distance <0.5 m; maintains FOV constraint (45∘). | [10] |
| Physics-guided gain regularization | Balboa 32U4 two-wheeled balancing robot | RNN (Elman) | Loss function penalizes deviation of neural sensitivities from real-world hardware gains. | Simulation-to-real transfer with Raspberry Pi 3B+ and Arduino hardware. | Hardware settling times match simulation; eliminated oscillations seen in naive Domain Randomization. | [11] |
| Optimized RNP Flight Procedure Execution | Fixed-wing aircraft (A320) | MHRL (Soft Actor-Critic base) | Compliance with RNP safety specs (altitude/velocity) and fuel consumption minimization. | JSBSim simulation under Dryden wind turbulence. | Successful RNP leg completion; 5% to 13% reduction in fuel consumption. | [12] |
| Rigid-Elastic Coupled Control (Hypersonic) | Hypersonic Flight Vehicle (HSV) | DRL (Barrier Lyapunov Function based) | Integrated constraints (attitude/vibration) into reward via Barrier Lyapunov Function (BLF) inequalities. | Numerical simulation. | Elastic vibration suppression increased to 85%; avoidance of control input saturation. | [13] |
| DL-NMPC-SD (Scene Dynamics) | Autonomous test vehicle (VW Passat) | IRL with Deep Q-Learning | Distance feedback minimizing energy trajectory to reference state waypoints. | GridSim virtual environment and real-world public road trials (150 km). | Lateral error (0.063 m) and heading error (4.51∘); outperforms DWA and End2End methods. | [14] |
| Viability-Based Action Shielding | Hypersonic vehicle (point-mass model) | Tabular Q-learning | Mode-dependent; penalizes tracking loss, control effort, and action switching. | Simulated hypersonic envelope (altitude, Mach, heating constraints). | Guaranteed hard safety constraint satisfaction; successful recovery maneuvers. | [15] |
| Unified RL Control System (URLCS) | Fixed-wing UAV | DDPG | Penalizes altitude error and pitch rate; weights time duration for smooth actuation. | MATLAB simulation using Aerospace Toolbox. | Steady-state error of 0.58 m; transient response time of 5 s (69% improvement over PID). | [16] |
| Integrated MPC–RL with Lipschitz filtering | Nonlinear aeroelastic wing system | Tabular Q-learning | Quadratic reward penalizing state deviation and actuator effort. | Simulation results and performance comparisons. | Overshoot (0.0069 m); plunge settling time (2.02 s); optimized RMS velocities. | [17] |
| Mode-based orbit insertion/station keeping | Rigid spacecraft | DQN with Eligibility Traces | Rewards proportional to distance from target orbit/thrust timing; penalties for state error. | Python-based simulation (OpenAI Gym) with Mars J2 perturbations. | 1.5' improvement over expert policy; outperformed rational agent in dispersed runs. | [18] |
| Autonomous self-separation (Free Flight) | Advanced Air Mobility (AAM) aircraft | Safe-DQN | Combination of goal Q-value and safe Q-value to ensure action safety. | Simulation-based testing against adversarial attacks. | Human-understandable actions; safety-boundary visualization. | [19] |
| Decentralized conflict resolution | Advanced Air Mobility (AAM) scenarios | Discrete SAC (SACD) | Attention-based encoding to prioritize specific intruders for avoidance. | Simulation environment (AAM-Gym). | Improved autonomous separation assurance in dense traffic. | [19] |
| Multi-agent distributed conflict resolution | UAV / Autonomous air mobility | PPO | Multi-agent coordination for distributed deconfliction. | Simulation studies. | Decentralized conflict resolution success in AAM scenarios. | [19] |
| Imitation of ATCO resolution advisories | UAV / Airspace users | DDPG | Reward defined by similarity to resolutions made by human Air Traffic Control Operators. | Interactive conflict solver simulation. | Successful learning of human-like deconfliction maneuvers. | [19] |
| Adaptive PID/PI Controller Tuning | Nonlinear systems / DC-DC Converter | Actor-Critic / Lightweight RL | Penalizes TD error (predictive vs reference), voltage deviation, and switching losses. | Numerical simulation and hardware validation (DSPs/MCUs). | Voltage overshoot reduced by 30%; TD error close to zero at steady state. | [20, 21] |
| Angular momentum control / FDD | Spacecraft Attitude Systems | Deep Learning / Machine Learning | Physics-based constraints for safety and mission-phase adaptivity. | Digital twin simulations and real telemetry. | Early detection of incipient faults; robustness under uncertainty. | [22] |
| Obstacle avoidance and navigation | Embedded hardware (Raspberry Pi/ESP32) | 1D-CNN (Transfer Learning) | Vibration-based fault pattern identification. | Real hardware and test benches. | Effective cross-domain generalization; millisecond-level latency. | [22] |
| Collision avoidance (Reynolds rules) | UAV quadrotor swarms | Not in source | Not in source | Simulation case study. | Deconfliction between UAVs in a swarm. | [19] |

[1] Sim-to-Real_Learning-Based_Nonlinear_MPC_for_UAV_Navigation_and_Collision_Avoidance_in_Unknown_Cluttered_Environments.pdf

[2] sensors-21-02534-v2.pdf

[3] drones-08-00475-v3.pdf

[4] 2404.00204v5.pdf

[5] aerospace-12-00134-v2.pdf

[6] Intelligent_Spacecraft_Attitude_Fault_Recovery_Using_Deep_Reinforcement_Learning.pdf

[7] research_proposal.pdf

[8] Ihsan final Proposal.docx

[9] study_the_proposla.pdf

[10] Intl J Robust Nonlinear - 2025 - Gu - Generalized Reinforcement Learning Enhanced Three‐Dimensional Impact Time Control.pdf

[11] Quantifying and Visualizing Sim-to-Real Gaps: Physics-Guided Regularization for Reproducibility - arXiv

[12] sensors-22-06475-v2.pdf

[13] vibration-09-00008.pdf

[14] Inverse_RL_Scene_Dynamics_Learning_for_Nonlinear_Predictive_Control_in_Autonomous_Vehicles.pdf

[15] Safety-Critical Reinforcement Learning with Viability-Based Action Shielding for Hypersonic Longitudinal Flight - arXiv

[16] Artificial_Intelligence_in_UAV_Flight_Controls_Deep_Reinforcement_Learning_Based_Altitude-Hold_Strategies_for_Fixed-Wing_UAVs.pdf

[17] 2512.12855v1.pdf

[18] AAS 19-447 SPACECRAFT DECISION-MAKING AUTONOMY USING DEEP REINFORCEMENT LEARNING - Dr. Hanspeter Schaub

[19] aerospace-13-00113-v2 (1).pdf

[20] Design of a Reinforcement Learning PID controller

[21] Edge-AI-Enabled Power Electronics with Embedded Intelligent Control

[22] electronics-15-00476.pdf